- V2G: optimale V2G-day-ahead-werte nur als Inputkurve

- non-convex MIQCP

- INPUTS: order_dataset

- AI statement am ende rein

- if a bev is charging PV generated energy it should be priced at the current private_charging_price_€/kWh because the PV energy could otherwise be sold at the private_charging_price_€/kWh —> opurtunitätskosten und kein fixer eigener PV generation price

- driver behaviour consideration can be toggled to compare regular human drivers and autonomous driving trucks

- externes Laden Strafe für zeit außer wenn über lenkzeit dann 45min Lenkzeitpause

- only one-day-operation, no multi-day trips and routes

- marcel paper zitieren: alle v2g use cases abgedeckt + vehicle-to-vehicle (https://www.sciencedirect.com/science/article/pii/S2352484726004257)

- The DSO demand cahrge of default 17€/kW is consistent with a German Leistungspreis in the under-2500-h/a utilisation band, where the demand charge is low and the Arbeitspreis carries more — which fits a depot whose grid draw is peaky. Just note the model prices energy at spot with no Arbeitspreis on top, so the network-fee side is now represented only by this term.

- Nachteile Betz et al: nicht-lineare Teile (SOC, Stromnetz, Energiemanagement) als linear angenommen

- Alterung: C-Rate vernachlässigbar weil immer ca max 1C siehe DIsstransfer 20.04.26

- Fahrer: aufteilen auf Fahrten, Fahrer Stundenlohn als Input in Optimierung

- price for charging at the depot with energy from the public grid is eneryg_spot_price_€/kWh + a new variable which is the sum of all overhead for buying (taxes, tarrifs, etc) that the user can set (default value 15c/kWh). price for charging at the depot with energy from the own PV at the depot is eneryg_spot_price_€/kWh - a new variable which is the sum of all overhead (selling costs of the seller platofrom, EEG) that the user can set (default value 0c/kWh) since charging from your PV is the price for the oppurtunity of selling this energy at the spot market. still consider the new efficiency values for charging and discharging

- Enable the possibility to chain trips together to a route during one operating day. implement it by allowing to chain a trip with the same starting location directly behind a trip with the same destination location. add a new varibale and field in the web interface for the location of the home depot. compare the trip start and destinaton locations with this depot location. V2G and local charging is only possible at this location. if a trip ends somewhere else add a new trip back to the home depot location or to the closest trip which does not start at the home depot. if a trip does not start at the home depot add a trip to the start location or chain it in the best possible route with other trips of that day. add a 10% uncertainty factor around every location so that close by locations are still treted as the same location.

- Battery Degradation:

  - Parameter als Eingabe konfigurieren: Preis, Garantie, battery etc in fleet_dataset.xlsx

  - Methodik: äquivalente Vollzyklen mit SoC-Gewichtung und DoD-Kurve, Kostenumrechnung über Anschaffungspreis + Hersteller-Batterie-Garantie

- Depot:

  - PV erzeugung über PVGIS und user-parameter in depot_dataset.xlsc und date

  - Lastgang des Standorts mit rein in optimierung

  - depot load profiles ohne pv einspeisung verwenden in depot_dataset

- V2G:

  - flexibilitätspreis-kurve als day-ahead-kurve als input für arbitrage

  - beliebige use-case preiskurve als zweiten input über Flex-Index statt einzelner Use-Cases mit Flexbändern von Darstellung siehe Koreaner Vortrag

  - peakshaving ist implementiert über DSO-Leistungspreis

  - use cases: peak shaving, eigenverbrauchsmaximierung, arbitrage, generel flexibility

- Externes Laden an öffentlichen Ladesäulen:

  - Ladekurven nicht implementiert, lineares Ladeverhalten weil großbe batterie in eLkw und sehr lang maximale ladeleistung

  - externes Laden implementieren mit hoher Strafe für Ladezeit außer wenn in 45min Lenkzeitpause oder weniger Zeit bis zum Routenziel benötigt

- Lokales Laden am eigenen privaten Depot:

  - Optimierung soll lokalen PV Strom über Preis bevorzugen mit Ladeleistungslimitierung der aktuellen PV-Produktion minus den aktuellen Depotverbrauch

  - Parameter der local charging stations als Input vorgeben in depot_dataset.xlsx

- Battery Degradation

  - max 1C also laderate egal

  - perfect temperature contol -> keine temperatureffekte